PASSWORD

The PASSWORD program is provided to allow the password of LPU-series

radios to be accessed and modified.

This program is designed for use on IBM or IBM-compatible computers with a

disk drive operating under DOS version 3.30 or compatible DOS.  The

computer must also be equipped with an RS-232 interface and an LAA 0705

interface (available from Bendix/King).

1.0	Connecting the LPU-series radio to the computer

Connect the radio to the computer using the LAA 0705 interface.

Use the appropriate commands to set the default disk and directory to

where PASSWORD.EXE resides.  Type: PASSWORD

If the RS-232 port on your computer is not configured as COM1 use the

arrow keys to move the cursor to the Change COM PORT option and press

<ENTER>.  Use the space bar to toggle the displayed COM port and press

<ENTER> to select it.

1.1	Read/Set Password

To view the password currently programmed into the radio, use the arrow

keys to move the cursor to the 'Read/Set Password' menu option.  This

menu option allows this password to be modified and retained to allow

easy programming into multiple radios.

The current password will be displayed in the edit field.  The password

status  (ENABLED/DISABLED) will also be displayed.  The following keys

can be used to modify the password:

Left, Right arrow keys	Moves cursor one character

<Backspace>		Moves the cursor to the first character

<Up arrow>		Clears password to all zeroes

<ENTER>			Stores password for later use

<ESC>			Exits to main menu, password not stored

<SPACE BAR>		Increments keyboard enterable passwords

by one.  See section 1.5

If the password loaded was shown to be DISABLED, it will automatically

be ENABLED when programmed into a radio.

1.2	Program Password

When this option is selected from the main menu, the password from the

Read/Set Password operation (if one was entered) is displayed in the

edit field.  Pressing <ENTER> will program the displayed password into

the radio and enable the PASSWORD feature.

The password can be modified as before with the following exception:

The <Up arrow> key will swap the current password programmed into the

radio into the edit field, regardless of what may have been entered

under the Read/Set option.  The <ESC> key will exit this function

without programming the password into the radio.

1.3	Disable Password

Pressing <ENTER> when this menu option is highlighted will disable

the password in the radio.  The password is still retained in the

radio's memory, but the radio will not prompt for the password when put

in programming mode.

1.4	Examples

To enable the password already stored in the radio:

Select 'Program Password'

Press 'S'   (puts current radio password into the edit field)

Press <ENTER>

To program multiple radios with the same password:

1  Select 'Read/Set password'

2  Change the password in the edit field as necessary

3  Press <ENTER>  (Stores the password for next step)

4  Select 'Program Password'

5  Press <ENTER>  (programs radio with password)

--To program additional radios with this password--

6  Press <ENTER>  ('Program Password' is already highlighted

at the main menu)

7  Press <ENTER>  (programs radio with password)

Repeat steps 6 and 7 (press <ENTER> twice) to program each additional

radio.

1.5	Disabling Keyboard Programming

To disable radio programming from the keyboard.  Enter any of the

following characters into the password:

C, D, E, F

Whenever one of these characters is present in the password in the

edit field, a warning will be displayed.  Note that a password

containing one of these characters cannot be incremented.