EX-1489 ICF-3 (Icom Cloning Format-3) cloning software ver. 1.1

COMMON DOCUMENT

Copyright 1991-1994 Icom Inc.

Portions of the software copyrighted

by Microsoft Corporation 1982-1988

This document file explains start up and shut down procedures as well as basic

operation of the EX-1489 cloning software. These procedures are the same

whether you are programming/cloning the IC-F30, IC-F40, IC-F30LT, IC-F40LT LMR

or PMR portables. For detailed instructions pertaining to the specifics of LMR

or PMR programming/cloning refer to the LMR.DOC and PMR.DOC files,

respectively.

**                                                   **

** IMPORTANT: Before using this program, make a      **

**            backup copy of the original disk and   **

**            use the backup copy as your working    **

**            copy. Store the original disk in a     **

**            safe place.                            **

**                                                   **

**                                                   **

** IMPORTANT: In order for this cloning software     **

**            to run properly, your DOS must be      **

**            configured with the following          **

**            parameter:                             **

**                                                   **

**               SET COMSPEC=C:\ COMMAND.COM         **

**                                                   **

**           (Refer to your DOS instruction manual.) **

**                                                   **

**                                                   **

** IMPORTANT: If your computer employs a "16550"     **

**            UART IC, it may hangup while cloning.  **

**            In this case, the FIFO function must   **

**            be turned OFF using commercially       **

**            available software.                    **

**                                                   **

- 1 -

-------------------------------------------------------------------------------

1. PROGRAM STARTUP

-------------------------------------------------------------------------------

This programming software contains 4 separate areas, enabling programming/

cloning of all Icom F30/F40 series transceivers. To start up a desired

area, 3 methods are available:

-Automatic startup according to data programmed in a connected transceiver;

-Startup by reading a previously saved file;

-Manual startup by specifying the desired program.

1) Boot up DOS.

2) Insert the EX-1489 backup disk.

3) Type the start command:

**                                                                **

**   ICC [Enter]                                                  **

**                                                                **

** (ICC is an abbreviation of Icom Cloning Format Control Center) **

**                                                                **

The Control Center screen appears.

4) Use the [Arrow] keys to select one of the program startup conditions:

a. Auto define

-Connect a transceiver using the OPC-427 cloning cable, then push

[Enter] to startup using the data set in the connected transceiver.

Make sure the cloning cable is connected to the proper serial port as

selected in "Serial Port" at the bottom of the screen.

-After the program starts up, use "PC<-TR" to read the transceiver's

basic data.

b. File

-Push [Enter] to indicate the previously selected file, use an [Arrow]

key to select it, then [Enter] again; or, push [Backspace], then type

a filename for direct selection of a desired file.

c. Manual define

-Push [Enter] to startup using the default settings for the IC-F30/F40

PMR, IC-F30LT/F40LT PMR, IC-F30/F40 LMR or IC-F30LT/F40LT LMR.  PMR

means a 5-Tone unit is either already installed or is optional.

- 2 -

-------------------------------------------------------------------------------

2. BASIC OPERATION AND SHUT DOWN

-------------------------------------------------------------------------------

The EX-1489 cloning software includes several screens for transceiver

programming. All screens or operations can be accessed from a TOP menu.  The

TOP menu appears when the [Esc] key is pushed. A submenu appears under the TOP

menu when an item in the TOP menu is selected with the [Arrow] and [Enter]

keys.

1) After the program starts up, push the [Esc] key to indicate the TOP menu.

2) Push a highlighted character; or the [Arrow] and the [Enter] keys to

select the desired item.

- A submenu appears under the selected item.

3) Push a highlighted character; or the [Arrow] and the [Enter] keys to

select the desired screen.

- The desired screen appears.

4) Select the desired item with an [Arrow] key, then push [Space] or the

digit keys to set the item.

- See the separate document files, LAND.DOC and PMR.DOC for screen

descriptions for the Land and PMR portables, respectively.

5) To return to the Control Center screen, select Quit from the File submenu

(data is not saved in this case); or, select Save from the File submenu

(data is saved).

-You may be prompted for a Yes(Y) or a No(N) before returning to the

Control Center screen (if the data in the file has been changed).

6) To exit the EX-1489 program and return to the system prompt, select Quit,

then push [Enter].

- 3 -