EX-1563  FIELD PROGRAMMING SOFTWARE

For the IC-A3

Copyright 1991-1994 Icom Inc.

Portions of this software are copyrighted by Microsoft Corporation 1982-1988

**                                                       **

**  This program is designed to perform cloning for the  **

**  IC-A3 VHF AIR BAND TRANSCEIVER only.                 **

**  Before using the program, make a backup copy of the  **

**  original disk.                                       **

**                                                       **

-------------------------------------------------------------------------------

TABLE OF CONTENTS

-------------------------------------------------------------------------------

1.  PROGRAM START UP AND SHUT DOWN ...................  2

2.  OPERATION ........................................  2

3.  SCREEN DESCRIPTIONS ..............................  3

COMMON SCREEN ..................................  3

MEMORY CHANNEL SCREEN ..........................  6

WX CHANNEL SCREEN ..............................  8

4.  CLONING OPERATION ................................  9

5.  FILE SAVING AND LOADING .......................... 11

6.  PRINTING ......................................... 13

- 1 -

-------------------------------------------------------------------------------

1. PROGRAM START UP AND SHUT DOWN

-------------------------------------------------------------------------------

1) Boot up DOS.

2) Insert the EX-1563 backup disk.

3) Type the start command:

**                                     **

**  EX1563 /(version) /expert [Enter]  **

**                                     **

1. USA or OTHER can be used as "(version)."

2. When the version is specified, the corresponding default settings

are read.

3. When no version is specified, the previously saved contents are

called.

4. When "/expert" is specified, all items in the COMMON screen are

adjustable; when "/expert" is not specified, some items in the

COMMON screen cannot be changed. (See p. 3)

5. When a file name is used instead of "(/version)," the specified file

is loaded at startup.

4) The COMMON screen appears.

**                                                              **

**  The loaded file name appears in the bottom left corner of   **

**  the screen.  "NONAME.ICF" appears when no data file is      **

**  loaded.                                                     **

**                                                              **

5) When programming is completed:

push [Esc] (indicates the TOP menu)

then [F] (selects file SUB menu)

then [S] (saves the set data; if not necessary skip this operation.)

then [Q] (quits the program)

-------------------------------------------------------------------------------

2. OPERATION

-------------------------------------------------------------------------------

The EX-1563 has a TOP menu and 3 screens for programming.  All screens or

operations can be accessed from the TOP menu. A SUB menu appears under the TOP

menu when the item in the top menu is selected with the [Arrow] and [Enter]

keys.  The TOP menu appears when the [Esc] key is pushed.

1) After the program starts up, push the [Esc] key to indicate the top menu.

2) Push a highlighted character; or the [Arrow] and [Enter] keys to select

the desired item.

- A SUB menu appears under the selected item.

3) Push a highlighted character; or the [Arrow] key and [Enter] keys to

select the desired screen.

- The desired screen appears.

4) Select the desired item with an [Arrow] key, then push [Space] or the

digit keys to set the item.

- See descriptions of each screen for details.

- 2 -

-------------------------------------------------------------------------------

3. SCREEN DESCRIPTIONS

-------------------------------------------------------------------------------

////// COMMON SCREEN //////

+[COMMON]---------+------------+------------+

| TITLE           |            |            |

+-----------------+------------+------------+

| RxEDGE L|U [MHz]| [108.000]  | [135.975]  |

| TxEDGE L|U [MHz]| [118.000]  | [135.975]  |

| ANL             |  OFF       | ---------- |

| BEEP            |  ON        | ---------- |

| KEY LOCK        |  OFF       | ---------- |

| USABLE ch       |  ALL       | ---------- |

| WX     ch       |  ENABLE    | ---------- |

| DIAL ACTION     |  CONTINUOUS| ---------- |

+-----------------+------------+------------+

**                                                                     **

** NOTE: When "expert" is specified during program startup (p. 2), the **

**       upper and lower Rx and Tx frequency edges can be changed, as  **

**       well as the "USABLE ch," "WX ch" and "DIAL ACTION" items.     **

**       Otherwise, they are fixed.                                    **

**                                                                     **

SCREEN SELECTION

1) Push [Esc] key once or twice to indicate the TOP menu.

2) Select "Screen" with the [S] key, or the [Arrow] and [Enter] keys.

- A sub menu appears under the TOP menu.

3) Select "Common" with the [C] key, or the [Arrow] and [Enter] keys.

- The COMMON screen appears.

RxEDGE L|U [MHz] ("expert" must be specified during program startup in order

to adjust this item.)

This item sets the lower and upper frequency values for receiving. The left

column is for the lower edge and the right column is for the upper edge.

Selectable value:     103.000 to 199.975 (MHz)

Selection method:     Type the desired frequency in directly, then

push [Enter]; or,

Push [Space] to toggle the default setting and

programmed setting.

**                                               **

** NOTE: Even though out-of-frequency values can **

**       be entered, the transceiver may not     **

**       accept such frequencies.                **

**                                               **

- 3 -

TxEDGE L|U [MHz] ("expert" must be specified during program startup in order

to adjust this item.)

This item sets the lower and upper frequency values for transmitting. The

left column is for the lower edge and the right column is for the upper

edge.

Selectable value:     118.000 to 199.975 (MHz)

Selection method:     Type the desired frequency in directly, then

push [Enter]; or,

Push [Space] to toggle the default setting and

programmed setting.

**                                               **

** NOTE: Even though out-of-frequency values can **

**       be entered, the transceiver may not     **

**       accept such frequencies.                **

**                                               **

ANL (Auto Noise Limiter)

This item toggles the ANL switch ON and OFF. When ON is selected, the ANL

switch activates the ANL function; when OFF is selected, the ANL key has

no function.

Selectable value:    ON or OFF

Selection method:    Push [Space] to toggle the value.

BEEP

This item toggles the transceiver's confirmation beep tones ON and OFF.

Selectable value:    ON or OFF

Selection method:    Push [Space] to toggle the selection.

KEY LOCK

This item toggles the key lock function ON and OFF. When ON is selected,

the key lock function can be activated; when OFF is selected, the key lock

function cannot be used.

Selectable value:    ON  or OFF

Selection method:    Push [Space] to toggle the selection.

USABLE ch ("expert" must be specified during program startup in order

to adjust this item.)

This item toggles the selectable channels. When "ALL" is selected, all

channels can be selected; when "MEMO ONLY" is selected, only memory channels

are selectable.

Selectable value:    ALL or MEMO ONLY

Selection method:    Push [Space] to toggle the selection.

- 4 -

WX ch (weather channels) ("expert" must be specified during program startup

in order to adjust this item.)

This item enables and disables the ability to receive VHF Marine weather

channels.

Selectable value:    ENABLE or DISABLE

Selection method:    Push [Space] to toggle the selection.

DIAL ACTION ("expert" must be specified during program startup in order

to adjust this item.)

This item sets the way in which the transceiver's rotary dial acts. When

"CONTINUOUS" is selected, dial rotation changes the frequency continuously

(including the MHz digits);

when "kHz ONLY" is selected, rotation of the dial changes only the kHz

portion of the frequency.

Selectable value:     CONTINUOUS or kHz ONLY

Selection method:     Push [Space] to toggle the selection.

- 5 -

////// MEMORY CHANNEL SCREEN //////

+[MEMORY ch]---+-----+---------+-----+-----+

|CH  |FREQUENCY|DISP | COMMENT |LOCK-| Tx  |

| No.|    [MHz]| SEL.|         |  OUT| INH |

+----+---------+-----+---------+-----+-----+

|  1 | 118.000 |  -  | [BLANK] |     |     |

|  2 | INHIBIT |  -  | [BLANK] |     |     |

|  3 | INHIBIT |  -  | [BLANK] |     |     |

~                                          ~

~                                          ~

| 48 | INHIBIT |  -  | [BLANK] |     |     |

| 49 | INHIBIT |  -  | [BLANK] |     |     |

| 50 | INHIBIT |  -  | [BLANK] |     |     |

+----+---------+-----+---------+-----+-----+

This screen allows you to set all memory channel contents.

SCREEN SELECTION

1) Push [Esc] key once or twice to indicate the TOP menu.

2) Select "Screen" with the [S] key, or the [Arrow] and [Enter] keys.

- A sub menu appears under the TOP menu.

3) Select "Memory" with the [M] key, or the [Arrow] and [Enter] keys.

- The MEMORY ch screen appears.

FREQUENCY

Memory channel frequencies are set here.

1) Select the desired channel in the FREQUENCY column with an [Arrow] key.

2) Enter the desired frequency directly or push [Space] to inhibit the channel.

-The specified frequency in the COMMON screen can be entered (the default

is 108.000 to 135.975 MHz).

-"INHIBIT" appears when inhibit is specified.

**                                               **

** NOTE: Even though out-of-frequency values can **

**       be entered, the transceiver may not     **

**       accept such frequencies.                **

**                                               **

DISP SEL.

Each memory channel can hold both a programmed frequency and a comment. This

item selects the priority of the display when the memory channel is selected

(i.e. whether the frequency or comment appears). Once a frequency or comment

is displayed, pushing [MR] on the transceiver toggles the display between

the frequency and comment. Note that a comment must be programmed in the

following item, otherwise this item has no meaning and "-" appears.

1) Select the desired channel in the DISP SEL. column with an [Arrow] key.

2) Push [Space] to toggle the selection.

-"<" indicates that frequency is displayed; ">" indicates that the

programmed comment is displayed.

- 6 -

COMMENT

6-character comments or "notes" can be programmed into selected memory

channels.

1) Select the desired channel in the COMMENT column with an [Arrow] key.

2) Enter the desired comment in directly or push [Space] to clear the

comment.

-Use "_" (under bar) to input a "space."

LOCKOUT

The selected channels are locked out during memory scan. This setting can also

be performed via the transceiver's front panel.

1) Select the desired channel in the LOCKOUT column with an [Arrow] key.

2) Push the [Space] key to select lockout or non-lockout.

- "L" appears when lockout is specified and no indicator appears when

lockout is canceled.

Tx INH

The selected channels can only receive; transmit is inhibited.

1) Select the desired channel in the Tx INH column with an [Arrow] key.

2) Push the [Space] key to select transmit inhibit or transmit non-inhibit.

-"I" appears when transmit inhibit is specified and no indicator appears

when transmit inhibit is canceled.

- 7 -

////// WX CHANNEL SCREEN //////

+[WX ch]---+

|CH  |LOCK-|

| No.|  OUT|

+----+-----+

|  1 |     |

|  2 |     |

|  3 |     |

|  4 |     |

|  5 |     |

|  6 |     |

|  7 |     |

|  8 |     |

|  9 |     |

| 10 |     |

+----+-----+

When weather channels are selected with the transceiver, the channels in

the screen table can be used.

SCREEN SELECTION

1) Push [Esc] key once or twice to indicate the TOP menu.

2) Select "Screen" with the [S] key, or the [Arrow] and [Enter] keys.

- A SUB menu appears under the TOP menu.

3) Select "WX   ch" with the [W] key, or the [Arrow] and [Enter] keys.

- The WX CHANNEL screen appears.

LOCKOUT

The selected channels are locked out during scan. This setting can also be

performed via the transceiver's front panel.

1) Select the desired channel in the LOCKOUT column with an [Arrow] key.

2) Push the [Space] key to select lockout or non-lockout.

- "L" appears when lockout is specified and no indicator appears when

lockout is canceled.

- 8 -

-------------------------------------------------------------------------------

4. CLONING OPERATION

-------------------------------------------------------------------------------

Programmed data can be cloned to an IC-A3 instantly.  All data is clone

sent from PC to transceiver (PC -> TR) or clone read from transceiver to PC

(PC <- TR) for approx. 15 sec.  A cloning cable, the OPC-478, is needed to

connect a PC's RS-232C port to the transceiver.

For convenience, when changing a small portion of the programmed contents,

clone read the data from the transceiver, modify the data, then clone send

it back to the transceiver.

CLONE SEND

The programmed data in the PC is copied to the transceiver.

1) Connect the OPC-478 between the PC's RS-232C port and transceiver's

[SP] jack.

2) Turn the PC and transceiver power ON.

3) Start up this program.

4) Select "Clone" in the TOP menu with the [C] key, or the [Arrow] and

[Enter] keys.

- Clone sub menu appears.

5) Select "PC -> TR" in the SUB menu with the [P] key, or the [Arrow] and

[Enter] keys.

- "Sending oooooo" appears in the bottom left of corner of the screen.

- "CL IN" appears in the transceiver display.

- When cloning is finished, "CL IN" disappears.

**                                                        **

**  NOTE: When the wrong port is selected as the RS-232C  **

**        port, "ERROR: CLONE error" appears in the       **

**        bottom left corner of the screen.  In this      **

**        case, check the selection as described on the   **

**        following page.                                 **

**                                                        **

6) Turn the transceiver OFF and ON again.

- Clone send is now complete.

- 9 -

CLONE READ

The pre-set data in the transceiver is copied to the PC.

1) Connect the OPC-478 between the PC's RS-232C port and transceiver's

[SP] jack.

2) Turn the PC and transceiver power ON.

3) Start up this program.

4) Select "Clone" in the TOP menu with the [C] key, or the [Arrow] and

[Enter] keys.

- Clone sub menu appears.

5) Select "PC <- TR" in the sub menu with the [T] key, or the [Arrow] and

[Enter] keys.

- "CL OUT" appears in the transceiver's display.

- When cloning is finished, the transceiver's display returns to

previous indication.

- When cloning is finished, "CL OUT" disappears.

- Turn the transceiver OFF and ON to return to the previous display.

RS-232C

When the connected PC has 2 or more RS-232C ports, the desired port can be

selected with this program.  When the PC has only 1 port, "PORT #1" must be

selected, otherwise, the cloning operation cannot be performed.

1) Select "Clone" in the TOP menu with the [C] key, or an [Arrow] and

[Enter] keys.

- Clone sub menu appears.

2) Select "RS-232C" in the SUB menu with the [R] key, or the  [Arrow] and

[Enter] keys.

- RS-232C port selection block appears.

3) Select the desired port number to connect the cloning cable with the

[Arrow] and [Enter] keys.

- Selected number is the same as the PC port number assignment.

- 10 -

-------------------------------------------------------------------------------

5. FILE SAVING AND LOADING

-------------------------------------------------------------------------------

PROGRAM INFORMATION

The EX-1563 contains the following:

EX1563.EXE    : Main program

AIR_USA.INI   : U.S.A version default settings

AIR_OTHR.INI  : Non-U.S.A. versions default settings

A3.DOC        : This document file

When starting up this program and no version name, such as "/USA" or "/OTHER",

is used, or an invalid version name is used, AIR_USA.INI is loaded.

FILE SAVING OPERATION

1) Push [Esc] to indicate the TOP menu.

2) Select "File" in the TOP menu with the [F] key, or the [Arrow] and

[Enter] keys.

- File sub menu appears.

3) Select "Save" in the sub menu with the [S] key, or [Arrow] and

[Enter] keys.

- A file name box appears and shows "FILE: noname.icf".

4) Type the desired file name to be saved.

- ICF is recommended as a file extension for loading convenience.

- When no file extension is used, ".ICF" is automatically added to the

file name.

- The TOP menu and the file name disappear and the previous menu appears.

**                                                                        **

**  SAVING NOTE:                                                          **

**                                                                        **

**  When starting up the program and no file name has been loaded,        **

**  "noname.icf" appears in the file name box.  In this case, direct      **

**  file name entry is possible ("noname" disappears automatically).      **

**  However, once a file is loaded or saved, file name entry does not     **

**  change the file name. ([Del] or [Ins] key must be used to change the  **

**  file name.)                                                           **

**                                                                        **

**  To indicate the previously saved file name, "*" (wild card) may be    **

**  helpful to check the file name.                                       **

**                                                                        **

- 11 -

FILE LOADING OPERATION

1) Push [Esc] to indicate the TOP menu.

2) Select "File" in the TOP menu with the [F] key, or the [Arrow] and

[Enter] keys.

- File sub menu appears.

3) Select "Load" in the SUB menu with the [L] key, or [Arrow] and

[Enter] keys.

- A file name box appears and shows "FILE: *.ICF".

4) Type the desired file name, then push [Enter].

Or, push [Enter] to indicate the pre-programmed file name, select the

desired file name with an [Arrow] key, then push the [Enter] key twice.

- The typed or selected file is loaded, then the file name box

disappears.

- The loaded file name appears in the bottom left corner of the screen.

DOS COMMAND

In the "File" sub menu, a "Dos" item appears.  This command pauses the

program so that a DOS command can be used.  When you want to return to

normal operation of the program, type "EXIT", then push [Enter].

- 12 -

-------------------------------------------------------------------------------

6. PRINTING

-------------------------------------------------------------------------------

The set data can be printed out for convenience and reference for subsequent

programming.  Only the currently displayed screen is printed out, therefore,

the printout operation must be performed for each screen desired.

1) Push [Esc] to indicate the TOP menu.

2) Select "Print" in the TOP menu with the [P] key, or the [Arrow] and

[Enter] keys.

- Print sub menu appears and shows "Current".

3) Push [Enter] again to print out the currently displayed data.

4) After printing is complete, the TOP and SUB menus disappear.

- 13 -