Modulation Attenuation

Compensation alignment balances the modulation sensitivity of the VCO and reference modulation (synthesiser low frequency port) lines. Compensation algorithm is critical to the operation of signalling schemes that have very low frequency components, e.g. PL, and could result in distorted waveforms if improperly adjusted. The compensation value must be set at seven frequencies across the frequency range and for every channel spacing supported by the radio.

If you have read the Help Text for this procedure and require further information, refer to Chapter 3 of the Service Manual for the GM350 radio.

-- HELP_VOLTAGE_LIMIT Voltage Limit Voltage Limit

The transmit voltage limit softpot sets the maximum power control voltage. All seven voltage limit softpots are tuned and programmed automatically when the Program button is pressed.