Per Radio

The Per Radio menu option gives the user access to the features that are enabled/disabled on a Per Radio basis and not selectively configured on a channel basis. Selection of the option opens the Per Radio window which contains the Per Radio features collected together in functional groups under the headings:

Function Buttons

Transmit

Receive

Ergonomics

Volume

Function Buttons

______

'A2' radio models are 4-channel capable with no display and have two function buttons. 'A3' radio models are 128-channel capable and have a 3-digit display. These radios have six function buttons.

Programmable options for the buttons are:

ALL MODELS:

MONITOR: Allows user to select between the channel's squelch modes, which are set on a per channel basis.

REPEATER TALKAROUND: Sets the transmit frequency to be the same as the receive frequency.

CALL 1: Transmits current channel's single tone 1.

CALL 2: Transmits current channel's single tone 2.

A3 MODELS ONLY:

SCAN A: Toggles scan operation. Uses scan list A.

SCAN B: Toggles scan operation. Uses scan list B.

NUISANCE DELETE: Deletes unwanted channels from scan list.

MEMORY CHANNEL 1: Selects first memory channel.

MEMORY CHANNEL 2: Select second memory channel.

EXTERNAL ALARM: User control of external alarm enable/disable.

CODED / CLEAR: Turns internal Option Board scrambler on/off.

KEY SELECT: Allows user to select required scramble key used by internal Option Board scrambler.

Transmit

_______

Transmit LED

This feature is enabled by checking the Tx LED check box. When enabled, the red LED is lit continuously while the radio is transmitting.

RF Power 1

Allows definition of the absolute power level in Watts that will be used when a channel is in use that has Power Level 1 selected.

RF Power 2

Allows definition of the absolute power level in Watts that will be used when a channel is in use that has Power Level 2 selected.

Transmit Time Out Timer

The transmit timeout timer limits the amount of time the user is allowed to transmit. If the user tries to transmit for longer than the transmit timeout timer duration, then the radio will stop transmitting and sound the button error alert until the PTT is released.

The TOT duration is set per radio. The feature is enabled on a Per Channel basis by checking the identified check box in the Per Channel window Transmit Options Area.

When the feature is enabled, the user's transmit time is restricted to the value entered into the Transmit Time Out Timer spin box.

(Time Out Timer) Pre Alert

The TOT Pre Alert is a medium-frequency alert sounded for 130 ms, five seconds prior to the expiry of the Time Out Timer.  The volume of the Pre Alert is selectable as Variable if not Disabled.

(Transmit Time Out Timer) Rekey Inhibit Time

This programmable period of time starts with the expiry of the transmit timeout timer. During this period, the user is prevented from transmitting. If an attempt is made, the button error alert is sounded until the relevant button, e.g. the PTT, is released.

On A3, 128-channel models, if a Single Tone selective call is received during the rekey inhibit period, the Rekey Inhibit Duration is cancelled to allow the user to answer the call.

Forced Monitor Mode

Forced Monitor forces the user to monitor a channel before being allowed to key the radio by the PTT or a Call button. If enabled, the button error alert will be sounded whilst the PTT/Call button is being pressed, to warn the user that he cannot transmit. On release of the button, the radio will enter 'autoreset', allowing the user to monitor the channel for any activity. Once in the autoreset mode, any subsequent attempt to key the radio will cause the radio to transmit (subject to transmit allowed criteria).

Forced monitor will not be applied if the radio is already monitoring the channel in autoreset (e.g.: Microphone off hook). It should be noted that force monitor cannot automatically be overridden by entering Monitor Mode 1 or Monitor Mode 2 because these modes are dealer programmable and may be set inconsistently with the requirements of forced monitor. e.g.: 'Reverse Monitor' is sometimes required by customers where receive squelch is carrier, and Monitor 1 squelch is Private Line.

The Forced Monitor Mode combo box enables the radio to be programmed to force monitor only if the channel is busy, i.e. carrier is present.

The Forced Monitor feature is enabled/disabled per channel but should not be enabled on channels which have auto reset disabled.

Forced monitor will not have any effect if the radio is 'monitoring the channel' when an attempt is made to key up the radio. The radio is considered to be 'monitoring the channel' under the following circumstances:

The radio is carrier squelched.

The radio is open squelched.

The radio is in autoreset. This can be after PTT dekey, entering timed hook, entering forced monitor after an attempt to key the radio when not monitoring the channel.

The radio is in permanent hook.

The radio has defeated signalling requirements by changing monitor mode.

Forced Monitor Alert

This is a continuous medium frequency tone sounded when the PTT is pressed and the radio is forced to monitor the channel before transmitting. The volume of the Alert is selectable as Variable if not Disabled.

Receive

_______

Channel Busy LED

When enabled, the red, front-panel LED flashes during receive (while carrier is present on the channel).

Autoreset Time

On entering into auto reset mode, the auto reset timer will be started. The radio will reset to the default channel squelch setting on expiry of the auto reset timer.

The auto reset mode will be entered for the following reasons:

1. On dekey of the radio (release PTT).

2. Detects the correct PL for coded squelch only channels. The auto reset timer will be started as soon as the PL frequency is detected (detecting PL whilst in the auto reset mode will not restart the timer unless carrier override is specified).

3. Going 'off-hook', when Hook (Per Radio Ergonomics) is programmed to be timed.

4. Being forced to monitor the channel before transmit by forced monitor.

5. For A3 models, if the radio is selectively called using single tone. The auto reset timer will be started as soon as the selective call has been received.

Channel Spacing

For 20/25kHz models ONLY, this feature allows the selection of channel spacing for the radio. It causes the radio to use factory programmed Alignment data for transmit deviation and receive squelch settings for either 20kHz or 25kHz channel spacing.

Options: 20kHz and 25kHz.

Note: For12.5 kHz radios, this feature is non-selectable.

Monitor 2 Alert

A high frequency alert tone sounded for 130 ms when monitor 2 squelch comes into effect (when the monitor button is pressed and held for 1 second). This feature may be enabled with Variable Volume only.

External Alarm Mode

This feature is available on 'A3' model display radios only. It is used to provide a signal on the accessory connector that can trigger a suitable relay to sound the vehicle's horn, or to switch on its lights, to inform a user, who may be away from his vehicle, that the radio has been selectively called.

The feature has the following options:

Disabled A selective call will not turn on the external alarm output signal.

Always (activated on selective call) A selective call will always activate the external alarm signal for the pre-programmed duration. The 'bell' icon will be displayed.

On Button The external alarm select option button is used to toggle between enable/disable for the external alarm signal. Therefore, the user may chose to turn off the signal when driving and only turn it on when the vehicle is parked.

External Alarm Duration

The length of time that the external alarm output is active after a call is received. NOTE:   External alarm is stopped if the radio is switched off, or if any of the radio buttons are pressed whilst the alarm is being asserted.

Ergonomics

_________

Power Up Mode

This feature determines the channel selected when the radio is powered up. The options available are Designated Channel and Last Channel. If the Last Channel option is selected and the radio was not on a particular channel but was in scan when powered down, the radio will power up into scan.

Designated Channel

Any one of the available channels in the radio that may be selected for the radio to power up onto.

Off Hook Function

Hook operation allows the user to toggle the squelch state. When on-hook, the radio squelch state is determined by the monitor mode and autoreset. When off-hook, the radio squelch state is determined by the programmed hook type. The Off Hook Function may be:

Disabled Going off-hook has no effect.

Permanent Going off-hook causes user to monitor channel until user goes back on-hook.

Timed Going off-hook causes user to monitor channel for the autoreset duration.

The required option is selected using the identified combo box.

Handset Audio

This check box must be enabled when a handset, not microphone, is connected. If the option is not selected, the loudspeaker audio will not be turned off when the handset is picked up, i.e. off hook. Also, it is recommended to program Permanent Hook.

Memory Channels 1 and 2

Two of the radio's option buttons can be programmed to select predetermined (memory) channels; these are identified as Memory Channels 1 and 2. The two Memory Channel spin boxes enable selection of the default channels.

Modifiable

Select either, or both, of these check boxes to enable the user to change the memory channel(s) through use of the programmed option button(s).

Power Up Alert

A high frequency alert tone sounded for 530 ms when the radio is switched on, to indicate that the power-up checks have been successful. This alert tone is selectable, the options offered are Disabled and Variable Volume.

Keypad Acknowledge Alert

A high frequency alert tone sounded for 130 ms when the user presses a valid button on the keypad. This alert  is selectable, the options offered are Disabled and Variable Volume.

Button Error Alert

A medium frequency alert sounded continuously as long as the error condition exists. This alert is sounded if a button is pressed when not permitted, or if the radio is changed to an undefined  or inhibited channel. This feature is selectable, the options offered are Disabled and Variable Volume.

Volume Select Alert

When the user is adjusting the radio speaker volume, the volume select alert is sounded for the duration of the volume button press unless the speaker is open for received audio. The volume select alert is a high- frequency continuous tone sounded at the received audio level. The alert may set to be on or off.

Call Alert

A high-frequency interrupted tone, 162 msecs on and 81 msecs off, sounded five times when the radio is individually called  by means of the single-tone encode/decode facility. This feature is selectable, the options offered are Disabled and Variable Volume.

Call Reminder Alert

A high-frequency, double tone, 162 msecs on and 81 msecs off and 81 msecs on, sounded at intervals after a Call Alert until the call is responded to. This feature is selectable, the options offered are Disabled and Variable Volume.

Volume

______

Variable Volume

This feature defines the level of the variable volume alert and allows an offset to be added to the normal volume setting when generating alerts.

Fixed Volume

This feature allows the level of fixed volume alerts to be set.

Minimum Volume

The volume is controlled using the volume up/down button and is increased and decreased by a fixed volume step change size. The volume select alert is sounded for the duration of the volume button press unless the speaker is open for received audio. The volume select alert is given at the received audio level.

Pressing the volume up/down button when at maximum or minimum volume will result in no volume change. If the volume up/down button is pressed and held, the radio will increase/decrease the volume continuously and the volume select alert will sound continuously, the alert increasing/decreasing in volume.

Use the Minimum Voltage spin box to set the minimum volume level that is allowed (for example, to prevent users in high noise environments from turning the volume down so far they would not hear a message).

Side Tone Volume

This feature defines the level of the variable volume side tones (e.g.: Call 1 single tone encode) and allows an offset to be added to the normal volume setting when generating side tones.

Mic Gain Adjust (dB)

This feature allows the output from the microphone to be altered to suit the customer.