MODIFICATIONS FOR PHILIPS FM92 E BAND TO THE AMATEUR 6M BAND

1.	Remove the PA board.

Rewind coils as per the attached list.

L122 & L114  If these coils are wound together with a common tap, then the list length applies. If these are Separate coils, then the following measurements apply L122 - 31/2 turns, L114 6 1/2 turns.

General observation: All coils are 25 - 30 % more turns than originally manufactured by Philips.

Substitute Capacitors as per the attached list

NOTE: C122, 123&128 are added in parallel with existing capacitors

C127 & 144 are added in parallel beneath the board. Lay the capacitors flat.

There Is an 18k? Resistor on the right hand end, below the large coil and near the hairpin link between the choke and another resistor. This should be replaced with a 33k resistor.

2.	 Remove the vco module.

NOTE: Ensure that all solder is removed prior to lifting the unit from the board.

Remove the side from the VCO.

The glue solder is ineffective.

Heat the VCO case to flow the solder on the hybrid board post.

Separate the board from the die cast housing.

Remove the offset frequency coil. This is the coil closest to the end of the board

NOTE: Use a sharp hobby knife or engraving tool to cut away the mounting cement from

the mounting legs.

Heat Glue for removal also.

Rewind the coil with 5 1/2 turns of the same diameter width 0.8 mm wire.

Space the coil to fit the existing hole centres

Resolder the coil to the board.

NOTE: Coat the offset coil with clear nail varnish after fitting and soldering

The offset coil slug is longer.

Ensure that both slugs move freely, Lubricate with silicon grease (Heat sink compound

will do) if necessary.

Insert 10 pF NPO Capacitors across both coils

Refit the board to the housing.

Refit the housing to the radio.

Resolder all of the connections.

3.	 Remove the front end tuning block.

NOTE: Ensure that all solder is removed from the pins prior to removal of the die cast housing.